Rehabelt System: Integration Manual

1. Overview

This system uses an "Edge AI" approach. The ESP32 collects movement data, buffers it into 2-second windows, and feeds it to the On-Board AI model. The AI tells you what the user is doing, so you can calculate how they are doing it.

The Workflow

- Input: Real-time IMU Data (Acceleration & Gyroscope).

- Process: The AI Model (model_data.h) classifies the movement.

- Output: * If Walking: Trigger Gait Metrics (Step Length, Sway).

If Sitting/Supine: Log state and rest.

2. Installation

Before writing any code, ensure your Arduino IDE has the required engine to run the model.

- Open Arduino IDE.

- Go to Sketch      Include Library      Add .ZIP Library…

- Search for EloquentTinyML.zip from the same folder provided to you

- Install the provided library

3. Project Setup

Your project folder must look exactly like this:

/Rehabelt_Firmware

├── Rehabelt_Firmware.ino    (The main code)

└── model_data.h                    (The AI model file)

Step: Copy the model_data.h file I provided into the folder where you save your sketch.

4. Researchers’ Instructions:

On SECTION A in the .ino code (code for ESP32)
- Note that the model expects inputs in m/s²

- Most MPUs output in g (Gravity).

- Replace the dummy variables with your actual mpu.getEvent() calls.

On SECTION B in the .ino code (code for ESP32)

- The function is triggered automatically only when the model detects “WALKING”

- Researchers have access to the variable sensorBuffer.

Ex. sensorBuffer[45][2] //To access the 45th sample on the Z-axis acceleration

sensorBuffer[i][0]	//Acc X (Lateral)

sensorBuffer[i][1]	//Acc Y (Vertical)

sensorBuffer[i][2]	//Acc Z (Forward)

sensorBuffer[i][3]	//Gyro X

sensorBuffer[i][4]	//Gyro Y

sensorBuffer[i][5]	//Gyro Z

Possible Issues:

EloquentTinyML.h: No such file	//Library is missing, need to install EloquentTinyML from Arduino Library Manager.

model_data was not declared	//Missing file, make sure that model_data.h is inside the folder with your .ino file.

Compilation error: line too long	// Bad model file, make sure to use the model_data.h that I provided.